Computer Science Department - ITU

Computer Networks Lab

| Course Instructor: Anam Zahid | Dated: 9/05/2024 |
| --- | --- |
| Lab Engineer: Ansa Niaz & Ghulam Ruqia | Semester: Spring 2024 |
| Session: 2023-2027 | Batch: BSCS2023 & BSAI2023 |

# Lab 12. Introduction of NAT(Network Address Transmission)

| Name | Roll number | Report Marks(10) | Viva Marks(5) | Total Marks (15) |
| --- | --- | --- | --- | --- |

Checked on: ____________________________

Signature: ____________________________

## Objective

The objective of this lab is to do some networking commands to help us better understand networking and, in its troubleshooting, as well as its administration.

## Equipment and Component

| Component Description | Value | Quantity |
| --- | --- | --- |
| Computer | Available in lab | 1 |

## Conduct of Lab

- Students have to perform this experiment on Wireshark.

- Students are required to perform this experiment individually.

- In case the lab experiment is not understood, the students are advised to seek help from the course instructor, lab engineers and  assigned teaching assistants (TAs)

- At the end of the lab, every student is required to save the completed lab manual in PDF format and submit this single PDF file on Google Classroom at the submission link created for this lab..

## Theory and Background

NAT is a networking technique that allows multiple devices within a private network to share a single public IP address. It works by translating private IP addresses of devices in the local network to a single public IP address when communicating over the internet. NAT enhances network security, conserves public IP addresses, and simplifies network management by hiding internal network structures from external networks.

Lab Tasks

In this lab, we’ll capture packets from a simple web request from a client PC in a home network to a www.google.com server. Within the home network, the home network router provides a NAT service. As in our other Wireshark labs, we collect a Wireshark trace on the client PC in our home network. This file is called NAT_home_side2. Because we are also interested in the packets being sent by the NAT router into the ISP, we’ll collect a second trace file at a PC (not shown) tapping into the link from the home router into the ISP network. (The hub device shown on the ISP side of the router is used to tap into the link between the NAT router and the first hop router in the ISP). Client-to-server packets captured by Wireshark at this point will have undergone NAT translation. The Wireshark trace file captured on the ISP side of the home router is called NAT_ISP_side.

- Download the zip file http://gaia.cs.umass.edu/wireshark-labs/wireshark-traces.zip and extract the files need for this lab.

- Open the NAT_home_side file and answer the following questions. You might find it useful to use a Wireshark filter so that only frames containing HTTP messages are displayed from the trace file. You need to answer the question:

- What is the IP address of the client?

- The client actually communicates with several different Google servers in order to implement “safe browsing.” The main Google server that will serve up the main Google web page has IP address 64.233.169.104. In order to display only those frames containing HTTP messages that are sent to/from this Google, server, enter the expression “http && ip.addr == 64.233.169.104” into the Filter: field in Wireshark .

- Consider now the HTTP GET sent from the client to the Google server (whose IP address is IP address 64.233.169.104). What are the source and destination IP addresses and TCP source and destination ports on the IP datagram carrying this HTTP GET? At what time is the corresponding 200 OK HTTP message received from the Google server?

- What are the source and destination IP addresses and TCP source and destination ports on the IP datagram carrying this HTTP 200 OK message?

- Recall that before a GET command can be sent to an HTTP server, TCP must first set up a connection using the three-way SYN/ACK handshake. At what time is the client-to-server TCP SYN segment sent that sets up the connection used by the GET sent? What are the source and destination IP addresses and source and destination ports for the TCP SYN segment? What are the source and destination IP addresses and source and destination ports of the ACK sent in response to the SYN. At what time is this ACK received at the client? (Note: to find these segments you will need to clear the Filter expression you entered above in step 2.)

In the following, we’ll focus on the two HTTP messages (GET and 200 OK) and the TCP SYN and ACK segments identified above. Our goal below will be to locate these two HTTP messages and two TCP segments in the trace file (NAT_ISP_side) captured on the link between the router and the ISP. Because these captured frames will have already been forwarded through the NAT router, some of the IP addresses and port numbers will have been changed as a result of NAT translation. Open the NAT_ISP_side. Note that the time stamps in this file and in NAT_home_side are not synchronized since the packet captures at the two locations.

- In the NAT_ISP_side trace file, find the HTTP GET message that was sent from the client to the Google server at which this was sent as recorded in the NAT_home_side trace file). At what time does this message appear in the NAT_ISP_side trace file? What are the source and destination IP addresses and TCP source and destination ports on the IP datagram carrying this HTTP GET (as recorded in the NAT_ISP_side trace file)? Which of these fields are the same, and which are different, than in your answer to question 3 above?

- Are any fields in the HTTP GET message changed? Which of the following fields in the IP datagram carrying the HTTP GET are changed: Version, Header Length, Flags, Checksum. If any of these fields have changed, give a reason (in one sentence) stating why this field needed to change.

- In the NAT_ISP_side trace file, at what time is the first 200 OK HTTP message received from the Google server? What are the source and destination IP addresses and TCP source and destination ports on the IP datagram carrying this HTTP 200 OK message? Which of these fields are the same, and which are different than your answer to question 4 above?